快速生成树协议配置实验步骤

连接交换机：使用指令telnet [设备IP地址]访问设备，并使用用户名user与密码b402b402进行连接。设备IP地址为172.16.[座位号].20X，其中X为设备号码，1、2为交换机，3、4为路由器。

步骤一：为PC1、PC2配置IP地址和掩码，按照图6-33将设备连接起来

- 查看两台交换机生成树的配置信息 display stp（华为交换机默认开启MSTP，需要输入指令undo stp enable来关闭）

- 除保持实验网卡连通外，切断其他网络链路，在没有主动通信的情况下，观察1~2分钟，会有广播风暴产生吗？

- 观察下列两种情况，哪种情况下包增长得更快？

  - 用PC1 ping PC2（带参数-t）

  - 在PC1或PC2上ping一个非PC1与PC2的IP（用参数-t）

- 在进行（3）的两种操作时，在交换机上不时查看MAC地址表display mac-address

拔下端口2的跳线，继续进行以下实验

步骤二：交换机A的基本配置

<S5720-02-1>system-view

Enter system view, return user view with Ctrl+Z.

[S5720-02-1]sysname switchA

[switchA]vlan 10

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchA-vlan10]name sales

[switchA-vlan10]quit

[switchA]interface GigabitEthernet0/0/3

[switchA-GigabitEthernet0/0/3]port link-type access

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchA-GigabitEthernet0/0/3]port default vlan 10

[switchA-GigabitEthernet0/0/3]quit

[switchA]port-group pg1

[switchA-port-group-pg1]group-member GigabitEthernet0/0/1 to GigabitEthernet0/0/2

[switchA-port-group-pg1]port link-type trunk

[switchA-GigabitEthernet0/0/1]port link-type trunk

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchA-GigabitEthernet0/0/2]port link-type trunk

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchA-port-group-pg1]port trunk allow-pass vlan 10

[switchA-GigabitEthernet0/0/1]port trunk allow-pass vlan 10

Info: This operation may take a few seconds. Please wait a moment...done.

[switchA-GigabitEthernet0/0/2]port trunk allow-pass vlan 10

Info: This operation may take a few seconds. Please wait a moment...done.

步骤三：交换机B的基本配置

<S5720-02-2>system-view

Enter system view, return user view with Ctrl+Z.

[S5720-02-2]sysname switchB

[switchB]vlan 10

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchB-vlan10]name sales

[switchB-vlan10]quit

[switchB]interface GigabitEthernet0/0/3

[switchB-GigabitEthernet0/0/3]port link-type access

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchB-GigabitEthernet0/0/3]port default vlan 10

[switchB-GigabitEthernet0/0/3]quit

[switchB]port-group pg1

[switchB-port-group-pg1]group-member GigabitEthernet0/0/1 to GigabitEthernet0/0/2

[switchB-port-group-pg1]port link-type trunk

[switchB-GigabitEthernet0/0/1]port link-type trunk

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchB-GigabitEthernet0/0/2]port link-type trunk

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchB-port-group-pg1]port trunk allow-pass vlan 10

[switchB-GigabitEthernet0/0/1]port trunk allow-pass vlan 10

Info: This operation may take a few seconds. Please wait a moment...done.

[switchB-GigabitEthernet0/0/2]port trunk allow-pass vlan 10

Info: This operation may take a few seconds. Please wait a moment...done.

步骤四：配置快速生成树协议

交换机A：

[switchA]stp enable

Warning: The global STP state will be changed. Continue?[Y/N]:y

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchA]stp mode rstp

Info: This operation may take a few seconds. Please wait for a moment...done.

交换机B：

[switchB]stp enable

Warning: The global STP state will be changed. Continue?[Y/N]:y

Info: This operation may take a few seconds. Please wait for a moment...done.

[switchB]stp mode rstp

Info: This operation may take a few seconds. Please wait for a moment...done.

测试：用两根跳线将两台交换机按照图6-33所示连接起来。将步骤一再做一遍，比较配置前后的实验效果。生成树协议起到什么作用？

步骤五：验证测试。在一台非根交换机上执行上述命令后过5s，使用display stp interface GigabitEthernet0/0/1命令和display stp interface GigabitEthernet0/0/1命令查看，判断哪一个端口处于丢弃状态？哪一个端口处于转发状态？

[switchA]display stp

[switchB]display stp

根据以上信息，判断根交换机是交换机A还是交换机B？根端口是哪一个端口？

步骤六：设置交换机的优先级

[switchA]stp priority 4096

步骤七：验证交换机A的优先级

[switchA]display stp

实验结果显示，当有2个端口都连在1个共享介质上时，交换机会选择高优先级(数值小)的端口进入转发状态,而低优先级（数值大）的端口进入丢弃状态。如果两个端口的优先级相同，则端口号较小的端口进入转发状态。

[switchB]display stp

比较与步骤1中(1)的查询结果有什么区别。

步骤八：验证交换机B的端口0/0/1和0/0/2的状态

[switchB]display stp interface GigabitEthernet0/0/1

[switchB]display stp interface GigabitEthernet0/0/2

请回答：(1) 交换机B的端口0/1处于什么状态？

(2) 交换机B的端口0/1的端口角色是什么？

(3) 交换机B的端口0/2处于什么状态？

(4) 交换机B的端口0/2的端口角色是什么？

步骤九：实验分析

- 记录经过步骤7后每台交换机的交换机生成树信息，并填入表6-4中。

交换机生成树信息：

表6-4 交换机生成树信息（1）

| 属性 | 描述 | 交换机A | 交换机B |
| --- | --- | --- | --- |
| Port Priority | 网桥优先权 |  |  |
| CIST Bridge | 本机网桥ID，后48位是网桥MAC地址 |  |  |
| CIST Root/ERPC | CIST总根交换设备ID/外部路径开销（从本交换设备到CIST总根交换设备的路径开销） |  |  |
| CIST RootPortId | 根端口 |  |  |
| Designated | Port Role为Designated Port的端口 |  |  |

- 如果交换机A与交换机B的端口0/0/1之间的链路down掉（使用配置命令shutdown或拔掉网线），验证交换机B的端口0/0/2的状态，并观察状态转换时间。

端口0/0/1链路down掉后查看交换机B的端口0/0/2：

[switchB]display stp interface GigabitEthernet0/0/2

说明交换机B的端口0/0/2从阻塞状态转换到转发状态，说明生成树协议此时启用了原先处于阻塞状态的冗余链路。状态转换时间大约2s。

判断上述结论是否正确。

- 记录此时每台交换机的交换机生成树信息，并与(1)比较，分析发生的变化。

- 当交换机A与交换机B之间的一条链路down掉时，验证PC1与PC2仍能互相ping通，并观察ping的丢包情况。

以下为从 PC1 ping PC2的指令：

C:\>ping192.168.1.20-t

拔掉交换机A与交换机B的端口0/0/1（或0/0/2）之间的连线，观察丢包情况。请拔线前确定哪个是根端口、哪个是阻塞端口，解析拔线后的丢包情况。

- 记录此时每台交换机的交换机生成树信息，填入表6-5并与(1)比较，分析发生的变化。

表6-5 交换机生成树信息（2）

| 属性 | 描述 | 交换机A | 交换机B |
| --- | --- | --- | --- |
| Port Priority | 网桥优先权 |  |  |
| CIST Bridge | 本机网桥ID，后48位是网桥MAC地址 |  |  |
| CIST Root/ERPC | CIST总根交换设备ID/外部路径开销（从本交换设备到CIST总根交换设备的路径开销） |  |  |
| CIST RootPortId | 根端口 |  |  |
| Alternate | Port Role为Alternate Port的端口 |  |  |

- 启动监控软件Wireshark,捕获BPDU，并进行协议分析。

【实验思考】

(1)请问该实验中有无环路?请说明判断的理由。如果存在,说明交换机是如何避免环路的?

(2)冗余链路会不会出现MAC地址表不稳定和多帧复制的问题?请举例说明。

(3)将实验改用STP协议,重点观察状态转换时间。

(4)在本实验中,开始时首先在两台交换机之间只连接一根跳线,发现可以正常ping通。此时在两台交换机之间多接一根跳线,发现还是可以继续正常ping通。请问此时有广播风暴吗?